Slide 1 — Title Slide

What to say

“Imagine living in a world where almost every digital device depends on software and microcontrollers.

Most modern clocks today use an Arduino or another programmable chip. They are easy to build, but you rarely see how the clock actually works internally.

My project, the TTL Clock, is different. It is a digital clock built completely with hardware logic instead of hidden software.”

Slide 2 — Pitch: The Need

What to say

“The problem with many modern digital projects is that the final result works, but the internal logic is often hidden inside software.

For example, with an Arduino clock, the microcontroller does most of the work. You write code, upload it, and the clock works. But it is difficult to see how the time is actually created, counted, and displayed.

For my project, I wanted something more technical and more visible. I wanted a project that shows how digital electronics really work step by step.

So the need behind this project is simple: I wanted to build a clock that makes the logic visible instead of hiding it behind programming.”

Slide 3 — Pitch: My Approach

What to say

“My solution is a clock built with TTL logic chips from the 74xx series.

TTL stands for Transistor-Transistor Logic. These chips use electronic logic gates and circuits to process digital signals.

Instead of using software, firmware, or a microcontroller, this clock works entirely through hardware logic.

Every second that appears on the display is created by real electronic circuits working together.

The project is based on the open-source TTL Clock by Sergey Kiselev. His project includes PCB designs, assembly instructions, a bill of materials, and hardware files for building a complete digital clock.”

Slide 4 — Pitch: How the Clock Works

What to say

“The clock starts with a 32.768 kilohertz crystal oscillator. This type of crystal is commonly used in clocks because it gives a very accurate timing signal.

That signal is divided step by step by logic circuits until it becomes a one-second pulse.

After that, the pulse goes into the counter circuits. These counters keep track of seconds, minutes, and hours.

Finally, decoder chips convert the values from the counters into readable numbers on the 7-segment displays.

So the process is: the oscillator creates the timing signal, the logic circuits divide it, the counters count the time, the decoders convert the signals, and the displays show the final time.”

Slide 5 — Pitch: Why My Project Stands Out

What to say

“The main advantage of this project is transparency.

You can physically follow how the signal moves through the system. You can see how the timing signal is created, how it is counted, and how it becomes visible on the display.

Compared to a standard Arduino clock, my TTL Clock is more educational from an engineering perspective.

My project stands out because it uses no microcontroller, it demonstrates real digital logic, it combines soldering skills with circuit understanding, and it shows how older computers and digital systems originally worked.

So this project is not just about building a clock. It is about understanding the foundations of digital electronics.”

Slide 6 — Demo Part 1: Power Supply and Oscillator

What to say

“Now I will start the demo.

First, I will show the power section of the clock. The TTL chips operate on a stable 5-volt power supply. Without stable voltage, the logic circuits and displays would not work correctly.

Next, this part is the crystal oscillator. It generates a frequency of 32.768 kilohertz, which is commonly used in digital clocks because it is accurate and easy to divide down into seconds.

The circuit divides this frequency step by step until it creates a one-second pulse.

That one-second pulse acts as the heartbeat of the clock. Every time this pulse happens, the clock can count one second forward.”

Slide 7 — Demo Part 2: Counters, Displays and Controls

What to say

“Next, the one-second pulse goes into the counter ICs.

These counters keep track of seconds, minutes, and hours. When the seconds reach 59, the logic resets them back to 00 and automatically increases the minute counter.

The same idea is used for the minutes and hours.

After the counters, the values are sent to BCD-to-7-segment decoder chips. These chips convert the binary signals into readable numbers on the display.

That is how the time becomes visible.

The clock also has buttons that allow me to adjust the hours and minutes manually. This shows that the clock is fully interactive, even though it does not use software.

As a backup plan, I also prepared screenshots, photos, or a short demo video in case the live demonstration does not work correctly.”

Slide 8 — Conclusion

What to say

“To conclude, the TTL Clock is more than a normal digital clock.

It is a practical demonstration of how digital electronics work at the hardware level. Instead of relying on software, this project uses pure logic circuits to generate, count, and display time.

Every function is performed by real electronic circuits. There is no programming, no firmware, and no hidden software.

That makes the project both technically challenging and educational.

My project proves that it is possible to build a fully functional digital system using only TTL electronics.

And that is what makes it different from most modern clock projects.

Thank you for listening. Are there any questions?”